Problem need to be solved:

- VCDL’s Behaviour change between pre and post simulation(solve by change the MOS size but should have a better way)

- Voltage reference is not stable when PI out change a lot(hard to describe shortly)

- Add a bit for IDAC and digital input part

- Just comes to my mind: The cap of probe is quite big(10pF?), which lead the power consumption of the unity gain buffer become big, even dominant the power consumption. If so, It not that easy to check internal voltage change(but output voltage and current is easy).

Good point:

- The whole system works pretty good at post layout. Not a big difference between pre and post only Vc will rise a little bit.(because parasitic cap, Control pulse width/Vc curve will change)

- Quite stable at corner simulation, only current accuracy get worse.

- The power consumption even reduced a little because parasitic cap on digital part(and current limit caused by metal routeing)

For one channal

1：Comparator：

- T_hl(time from high to low)

- T_lh(time from low to high)

- Power

2:  VCDL:

- Delay/Vc Curve

- Power

3:	OTA for PI controller

- Power

- Ac analysis

4:	OTA for Current mirror

- Power

- Ac analysis

5:	Level Shifter:

- T_hl(time from high to low)

- T_lh(time from low to high)

- Power

6:	Test combine comparator/VCDL/NAND gate and LS

- The pulse width applied on rectifier gate(NMOS and PMOS) / Vc

7:	Switch:

- Efficiency at different load(Voltage needed)

- Power

8:	H_bridge:

- Efficiency at different load(Voltage needed)

- If FWD and REV works fine

9:	PI controller

- AC analysis

- Slew rate

10: 	IDAC

- Current accuracy

- Output resistance

11: Two assigner and Idle controller:

- If it works fine

BIAS BLOCK

1: Accuracy of bias voltage

2: The problem of V_REF_OUT

DATA_INPUT

1: Power

2:If it works